ATTACHMENT-5

INFORMATION DOCUMENT ABOUT THE SCIENTIFIC PAYLOADS

The teams competing in the TEKNOFEST Rocket Competition may carry out a scientific mission(s) as long as their payload is not mandatory (optional).

Within the scope of the TEKNOFEST Rocket Competition, scientific missions can be carried out at three (3) main difficulty levels (Simple, Medium and Complex Levels). Teams that complete the scientific task partially or fully successfully will earn additional points, depending on the difficulty level.

- Indispensable Requirements for Scientific Payloads

  - The teams can design scientific payloads (the difficulty level will be determined by the Competition Committee, considering the documentation submitted by the teams) based on the sample applications presented in this document (given below).

  - The teams will present all criteria(s) for measuring the success of the scientific mission, as well as the general and detailed (including sub-breakdowns) performance scoring that will form the basis for the assessment of performance, at the basic level in the PDR and in detail in the CDR. Taking into account the proposals of the teams, the final success criteria and all the criteria that will form the basis of the scoring will be determined by the Competition Committee.

  - Teams will share the basic purpose of the scientific payload, the task(s) to be performed, technical information regarding its design, visual(s), mathematical model(s), etc. at the basic level in the PDR and in detail in the CDR.

  - The final readiness of the scientific workload must be reported to the Competition Committee at the LRR stage, and all components must be proven to be operational.

  - For the successful handling of the payloads in the competition, the scientific data they produce must be transmitted uninterruptedly to the team’s ground station. This data must also be stored on the scientific payload and delivered to the Competition Committee, as even if the scientific mission is fully successful, the total success will only be considered partial.

  - The functionality and security evaluations of scientific payloads will be carried out separately by the Competition Committee at the competition site.

  - Teams are not allowed to use any hot/cold gas generators or pyrotechnic materials in their scientific payload designs.

- Difficulty Levels of Scientific Payloads and Sample Applications

  - Simple Scientific Payloads

    - Collecting Atmospheric Data Throughout the Flight

Temperature, density and pressure values vary depending on altitude in the atmosphere. The scientific payload to be developed by the teams should take measurements of ambient temperature, density and pressure values at a rate of at least 5 Hz (5 data per second) from the separation at the peak of the flight (apogee) until the safe landing via parachute on the ground, and shall transmit such data to the team’s ground station.

    - Collecting Static Pressure and Temperature Data from Various Points on the Rocket

During the flight of a rocket, different pressure and temperature levels develop at various points on the body. Thanks to the pressure and temperature distribution that occurs under real flight conditions, the validity of the models used in the design phase and the accuracy of the calculations can be tested. Using systems they will design, teams will be able to prove the pressure and temperature changes at different points on the rocket body and at different flight times based on measurements taken throughout the flight.

  - Medium Difficulty Scientific Payloads

    - Vibration/Load Damping Test

Rockets are exposed to high acceleration during flight. Such accelerations can put structural parts under strain. Teams may opt to design a vibration/payload-damping component/piece of equipment. The teams can prove mission success by transmitting data on the vibration/load damping effects of the components/equipment they have designed (data obtained through measurements made during the flight) to the team’s ground station.

    - Data Collection on Stress Loads on the Rocket Body

Rockets can experience high acceleration loads during flight, inflicting strains on the rocket’s structural parts. The teams can design a payload that tests the effects of strain on the rocket’s structural components, and transmit the acquired data to the team ground station to prove mission success.

  - Complex Scientific Payloads

    - Observer UAV (Unmanned Aerial Vehicle) Transport Task

Teams can develop Unmanned Aerial Vehicles (UAVs) for reconnaissance and surveillance. When the rocket reaches the apogee of its flight trajectory, the UAV will separate from the rocket and glide until it reaches the target point while obtaining image data. Teams can prove mission success by transmitting the images obtained by the UAV to the team ground station.

    - Fuel Tank Turbulence Test

The turbulence of the fuels in the fuel tanks of the liquid propellant rocket engines throughout the flight puts chaotic loads on both the fuel tanks and other structural elements, in addition to causing instability in rocket engine performance. Teams may opt to design an experimental setup for the observation and testing this phenomenon. The teams can prove mission success by transmitting the data they obtain from the turbulence test setup (based on measurements made throughout the flight) to the team’s ground station.